Soil Calibration Notes (assumes soil at steady state so it doesn’t work for regrowth)

Start on the Nitrogen side.

- N deposition=N leaching determines aqueous concentrations of N.  Adjust sorbed nutrient parameters (Vsnh4max and nh4) to adjust N leaching.  See #4 if this doesn’t work.

- This will affect N uptake so compensate using uptake maximum parameters. The trick seems to be getting Vsnh4max and nh4 consistent with each other so that the model doesn’t start out with 1000 umol N/L for the first couple of days of the year and then drops down to 1 or less.

- Adjust leaching and uptake simultaneously because they interact.  Don’t get too fussy getting N deposition = N leaching or Cum. Litter N = N uptake the next step will probably mess them up. (Hypothesis: If Naq doesn’t change drastically during the course of the year then the next step may not change N leaching and N uptake that much.)

- Adjust the ratio of mineralization/immobilization.  The easiest was to do this is to adjust MNA and INH4 (parameters aA and VINH4max) but there is also the path through humus.  So check the humus mineralization compared to the active mineralization.  Shift more N through humus using qH or aEI (fluxes FCEI and FNAH) but be careful not to shut off immobilization completely (Compare active soil C:N, qSA, to qH). Also check the Cum. Litter C and Cum. Soil Resp to make sure they are close (don’t worry if they don’t match yet). If there is a major discrepancy between Cum Litter C and Cum. Soil Resp then it’s necessary to adjust Humus resp (aH), which will effect N mineralization from humus. Check N uptake and N leaching, if they are no longer close go back to step 1 otherwise continue to step 5.

- Once happy with the rough mineralization/immobilization ratio then adjust MNA and INH4 again (should require only a slight change) to get Net N min = N uptake.

Now the Carbon Side

- Adjust Fcse until Cs is at steady state.

- Then adjust Fcei and Fcie to get Ci and Ce.

- Check Cum litter C = Cum. Soil Resp.